System Info as follows:

IBM Netfinity 5k5 Server with S3 Trio64V2 (1Mb) video adapter (this was probed correctly during Xconfigurator install process)

Monitor: Viewsonic E70 17” (couldn't be probed - I suspect use of a KVM Switch may be reason?  However, info relating to monitor was manually provided during install process)

Xserver Message:

Module Loader Present

OS Kernel: Linux version 2.4.18-14 (bhcompile@stipples.devel.redhat.com)

(default setting) LogFile: "/var/log/XFree86.0.log", Time: Thu Nov 20 2003

(default setting) Using ConfigFile: "/etc/X11/XF86Config"

(EE) error: No Devices Detected

Fatal Server Error: no screens found

XF86Config File Info:

Section "ServerLayout"

Identifier 		"Anaconda Configured"

Screen 		0 "Screen0" 0 0

InputDevice 	"Mouse0" "CorePointer"

InputDevice 	"Mouse1" "SendCoreEvents"

InputDevice 	"Keyboard0" "CoreKeyboard"

EndSection

Section "Monitor"

Identifier 		"Monitor0"

VendorName 	"Monitor Vendor"

ModelName 	"Monitor Model"

HorizSync 		30 - 70

VertRefresh 		50 - 120

Option 		"dpms"

EndSection

Section "Device"

Identifier 	  "S3 Trio64V2 (generic)"

Driver 	  "s3"

VendorName "S3 Trio64V2 (generic)"

BoardName   "S3 Trio64V2 (generic)"

#BusID

EndSection

Section "Screen"

Identifier 	"Screen0"

Device 	"S3 Trio64V2 (generic)"

Monitor 	"Monitor0"

DefaultDepth 16

SubSection "Display"

Depth 16

Modes "800x600" "640x480"

EndSubSection

EndSection

Section "DRI"

Mode 0666

EndSection